Justin Valentine

Jason Ross

Dalton Ellis

Ken Aoyagi

ENGR114 Fall T/R

12/7/2019

Balancing Car

Hardware Setup:

Bill of Materials:

| Part Name | Purpose | Item Name | URL | Price |
| --- | --- | --- | --- | --- |
| Redboard | Microprocessor | Arduino Uno | https://www.sparkfun.com/products/15123 | $20 |
| Arduino and breadboard holder | Base for breadboard | Arduino and breadboard holder | # https://www.sparkfun.com/products/11235 | 3.95 |
| Motor driver | Controls voltage levels to motors | # Dual TB6612FNG | # https://www.sparkfun.com/products/14450 | 5.45 |
| accelerometer | Sensing pitch level | MPU 92/65 | https://www.banggood.com/buy/mpu-9265.html | 12 |
| Hobby gearmotor | Moves vehicle | DG01D | https://www.sparkfun.com/products/13302 | 4.95 |
| Wheels | Provide rotational inertia and minimal points of contact to board | Wheel | https://www.sparkfun.com/products/13259 | 2.95 |
| Seesaw | Tilt car | Seesaw | HOMEMADE | N/A |
| Jumper wires | Facilitates flow of electrons with minimal resistance | Jumper wires | https://www.sparkfun.com/products/11026 | 2.25 |
| Mini power switch | Turns off power | Mini power switch SPDT | https://www.sparkfun.com/products/102 | 1.50 |

Hookup Guide:

See image below: